AI-Enabled Digital Twin for Aero Piston Engine Health Monitoring

Project Documentation

1. Problem Statement

MALE (Medium Altitude Long Endurance) UAVs rely on aero piston engines for long-duration ISR, surveillance, and defence missions. Conventional engine monitoring is threshold-based and reactive — it flags failures only after they occur, with no ability to predict degradation, estimate remaining useful life, or simulate mission-wise engine behavior.

This project develops the AI/ML core of a Digital Twin framework for aero piston engines: a system that continuously assesses engine health from sensor telemetry, flags abnormal conditions before they become failures, and — where possible — identifies the likely fault category.

2. The Dataset Availability Gap (and how we addressed it)

No public dataset exists specifically for MALE UAV piston engines — that data is either classified or held privately by OEMs. Rather than working with a mismatched proxy silently, we made this gap an explicit part of the project design:

- NASA C-MAPSS (turbofan degradation simulation) was evaluated early as a possible benchmark, but rejected as the primary dataset: it models jet turbofan engines, not piston engines, and lacks the sensor channels (CHT, EGT, oil pressure) relevant to this problem.

- NGAFID (National General Aviation Flight Information Database) — specifically the Large-Scale Annotated Aviation Maintenance Dataset (Yang et al., 2022) — was selected instead. It is real (not simulated) flight data from general aviation piston aircraft, with the correct sensor domain and real annotated maintenance events.

This is documented here deliberately: understanding why a given dataset is or isn't appropriate for a defence-grade UAV system is treated as part of the deliverable, not a footnote.

3. Dataset Description

Source: NGAFID Aviation Maintenance Dataset (Zenodo DOI: 10.5281/zenodo.6624956; Yang et al., arXiv:2210.07317)

Structure:

- flight_header.csv — 11,446 flights, one row each, with columns: Master Index, before_after, date_diff, flight_length, label, hierarchy, fold, class, target_class, hclass, number_flights_before

- flight_data.pkl — a dictionary keyed by Master Index, each value a raw per-second sensor array of shape (n_seconds, 23)

- stats.csv — reference statistics per operating-condition cluster, used to recover the 23 sensor column names (volt1/2, amp1/2, FQtyL/R, fuel flow, oil temp/pressure, RPM, 4x CHT, 4x EGT, OAT, IAS, VSpd, NormAc, AltMSL)

Labels:

- label / class / hclass — the maintenance issue type at increasing granularity (19 specific issue types, e.g. "intake gasket leak/damage", "rocker cover leak", collapsing into 4 broader hclass categories)

- before_after, date_diff, number_flights_before — proximity of a given flight to a real recorded maintenance event

- fold — a pre-defined 5-fold cross-validation split provided with the dataset, used throughout this project instead of a random split, to avoid data leakage across the same aircraft's flights

Class balance: heavily imbalanced. hclass == 0 (normal) accounts for ~81% of flights; the rarest fault category has only 75 flights total.

4. Data Quality Issues Found and Resolved

Two genuine data-engineering issues were identified and fixed during this project, both worth documenting as they affected result validity:

- Numeric overflow during feature computation. Raw sensor arrays are stored in a narrow floating-point format. Computing statistics (mean, std) directly caused silent inf overflow for ~99% of flights on several sensor channels (AltMSL, EGT, RPM, etc.). Fixed by casting to float64 before any statistical computation.

- Fold-direction error. An initial implementation trained on the smaller portion of the provided 5-fold split and tested on the larger portion — the reverse of correct practice. Corrected to train on 4 folds and evaluate on the held-out fifth.

All results below reflect the corrected pipeline.

5. Methodology

5.1 Feature Engineering (Baseline)

Per flight, per sensor: mean, std, min, max, and a coarse trend (last-third average minus first-third average) — ~115 features per flight. Fed into a Random Forest classifier as a first baseline.

5.2 Sequence Modeling (Primary Approach)

Statistical summaries discard the shape of a sensor trace over time — which, per this dataset's own published benchmarks, is where most of the fault signal lives. We moved to a 1D Convolutional Neural Network operating directly on each flight's raw sensor sequence:

- Each flight's raw sequence (variable length, ~3,400–5,200 seconds) is cleaned (inf→NaN, gaps interpolated) and resampled to a fixed 1,000-point sequence per flight, preserving the full flight's shape rather than truncating it.

- A 3-layer 1D-CNN (32→64→128 channels, batch-normalized, global average pooled) trained on GPU (NVIDIA RTX 5060, PyTorch/CUDA 12.8).

5.3 Two-Stage Pipeline

- Stage 1 — Fault Detection (binary): normal vs. any-fault, trained on all flights, with pos_weight-adjusted loss to address class imbalance.

- Stage 2 — Fault-Type Classification (multi-class): trained and evaluated only on flights with a known fault (hclass != 0), classifying which of the 4 broad fault categories applies. This mirrors a realistic operational flow: stage 2 only needs to run once stage 1 has flagged something.

6. Results

Stage 1 — Fault Detection

| Metric | Random Forest (statistical features) | CNN (raw sequence) |
| --- | --- | --- |
| ROC-AUC | 0.63 | 0.67 |
| Fault recall @ 0.5 threshold | 0.05 | 0.77 |

The CNN substantially improves recall at the cost of more false positives — a tunable tradeoff (see threshold sweep below), not a flaw.

Threshold sweep (CNN, stage 1):

| Threshold | Fault Recall | Fault Precision | Macro F1 |
| --- | --- | --- | --- |
| 0.3 | 0.94 | 0.21 | 0.32 |
| 0.4 | 0.88 | 0.23 | 0.41 |
| 0.5 | 0.77 | 0.25 | 0.50 |
| 0.6 | 0.54 | 0.28 | 0.56 |
| 0.7 | 0.26 | 0.29 | 0.56 |

Chosen operating point: 0.5, on the grounds that in a safety-relevant monitoring context, a missed fault (false negative) is materially more costly than a false alarm (false positive) — a false alarm costs an inspection; a missed fault risks the mission. The threshold is exposed as a configurable operator setting, not hardcoded.

Stage 2 — Fault-Type Classification

Trained on 2,136 fault-only flights (1,703 train / 433 test):

| hclass | Support (test) | Precision | Recall | F1 |
| --- | --- | --- | --- | --- |
| 1 | 242 | 0.67 | 0.55 | 0.61 |
| 2 | 48 | 0.26 | 0.33 | 0.29 |
| 3 | 129 | 0.53 | 0.40 | 0.45 |
| 4 | 14 | 0.13 | 0.71 | 0.22 |
| Macro avg | 433 | 0.40 | 0.50 | 0.39 |

Performance is meaningfully above the random-guess baseline (ln(4) ≈ 1.39 loss reduced to ~0.90 in training) but is not production-grade — consistent with the NGAFID dataset's own published benchmarks, which report fine-grained fault-type classification as an open, currently unsolved problem even for deep learning approaches on this exact data.

7. Known Limitations

- Detects presence and broad category of a fault; does not yet predict Remaining Useful Life (RUL) as a continuous countdown.

- Evaluates each flight independently; does not yet track degradation trends across an aircraft's flight history.

- Trained on general aviation piston aircraft data as a proxy — not MALE UAV-specific engine data, which does not exist in public form.

- Rare fault categories (as few as 75 total flights) limit achievable classification accuracy regardless of model architecture — a data scarcity ceiling, not a modeling deficiency.

8. Roadmap Beyond This Prototype

- Physics-based thermodynamic engine simulator to generate UAV-specific piston engine behavior and inject the specific fault modes (misfire, injector abnormality) not present in any public dataset.

- Per-aircraft trend modeling across sequential flights, feeding a genuine RUL estimate rather than per-flight binary detection.

- Transfer-learning experiments using C-MAPSS-pretrained sequence encoders, fine-tuned on piston-engine data.

- Edge-deployable lightweight model variant for onboard/GCS deployment, as distinct from the heavier ground-station research model.

- Visualization dashboard and mission-replay capability (see companion deliverable).

9. Technical Stack

Python, pandas, NumPy, scikit-learn, PyTorch (CUDA 12.8), trained on NVIDIA RTX 5060 Laptop GPU. Dataset: NGAFID Aviation Maintenance Dataset (CC BY 4.0).